Sentinel Network – Propellant Results

Per-satellite wet mass:        442.0 kg

Xenon allocation:              50.0 kg

Theoretical Δv capability:     2590 m/s

Nominal annual Δv budget:      5.5 m/s

Storm penalty:                 15.0 m/s every 2 yrs

Formation correction Δv:       5.0 m/s

Xe used (formation):           0.10 kg

Xe remaining for operations:   49.90 kg

==============================================================================

SENTINEL NETWORK v4.6 — PROPULSION-LIMITED SERVICE LIFE (FINAL)

==============================================================================

Propulsion-limited lifetime:    50 years

Total Xe used per satellite:    13.22 kg

Unused Xe margin:               36.78 kg

Fraction of Δv utilized:        26.4%

Constellation xenon cost:       ~$84.0M

NOTE: Lifetime limited by propulsion ONLY.

Power, radiation, avionics determine real mission life.

==============================================================================