Sentinel Network – AI Radiation Script

import numpy as np

import matplotlib.pyplot as plt

# ============================================================

# SENTINEL NETWORK v4.6 — RADIATION-ADAPTIVE AI & CNFI CONTROL

# PDR-GRADE | TRACEABLE | LOCKED

#

# PURPOSE:

#   Demonstrate deterministic throttling and safing of onboard

#   AI compute and AI-triggered sensors (CNFI) during radiation events.

#

# IMPORTANT:

#   - CNFI is a SENSOR, not compute.

#   - CNFI is enabled only when AI is enabled.

#   - This is a control-logic demonstration, not a radiation physics model.

#

# LOCKED REFERENCES (v4.6):

# - MARCI:   6 W (always on)

# - EMIRS:  22 W (always on)

# - Liulin:  3 W (always on)

# - CNFI:    8 W (AI-gated sensor)

# - AI GPU:  700 W full / 350 W throttled / 0 W safe

# - Bus + avionics explicitly modeled

# ============================================================

# ---------------- Power Constants (v4.6 LOCKED) ----------------

BUS_AVIONICS_W = 141.0   # Avionics, comms, ADCS, thermal, margins

MARCI_W  = 6.0

EMIRS_W  = 22.0

LIULIN_W = 3.0

CNFI_W   = 8.0           # Cognitive Narrow-Field Imager (AI-triggered)

GPU_FULL_W      = 700.0

GPU_THROTTLED_W = 350.0

GPU_SAFE_W      = 0.0

# Always-on sensor load

ALWAYS_ON_SENSOR_W = MARCI_W + EMIRS_W + LIULIN_W

# ---------------- Radiation Environment ----------------

GCR_BASELINE = 1.0

ELEVATED_RAD = 10.0

SEP_MULT     = 500.0     # 1972-class SEP proxy (SPENVIS heritage)

TIME_STEP_MIN = 1.0

minutes = np.arange(0, 1440, TIME_STEP_MIN)  # 1 sol

radiation = np.ones(len(minutes)) * GCR_BASELINE

SEP_START_MIN = 700

SEP_DURATION_MIN = 120

radiation[

(minutes >= SEP_START_MIN) &

(minutes < SEP_START_MIN + SEP_DURATION_MIN)

] *= SEP_MULT

# ---------------- Adaptive AI + CNFI Control Logic ----------------

gpu_power  = np.zeros(len(minutes))

cnfi_power = np.zeros(len(minutes))

ai_mode    = []

for i, rad in enumerate(radiation):

if rad <= ELEVATED_RAD:

# Nominal background radiation

gpu_power[i]  = GPU_FULL_W

cnfi_power[i] = CNFI_W

mode = "FULL AI"

elif rad < SEP_MULT:

# Elevated radiation → throttle compute, keep CNFI

gpu_power[i]  = GPU_THROTTLED_W

cnfi_power[i] = CNFI_W

mode = "THROTTLED AI"

else:

# SEP safe-mode

gpu_power[i]  = GPU_SAFE_W

cnfi_power[i] = 0.0

mode = "SAFE MODE"

ai_mode.append(mode)

ai_mode = np.array(ai_mode)

# ---------------- Total Spacecraft Power ----------------

total_power = (

BUS_AVIONICS_W +

ALWAYS_ON_SENSOR_W +

gpu_power +

cnfi_power

)

# ---------------- Report ----------------

modes, counts = np.unique(ai_mode, return_counts=True)

mode_times = {m: c * TIME_STEP_MIN for m, c in zip(modes, counts)}

print("=" * 80)

print("SENTINEL NETWORK v4.6 — RADIATION-ADAPTIVE AI & CNFI CONTROL (LOCKED)")

print("=" * 80)

for mode, mins in mode_times.items():

print(f"{mode:<16}: {mins:5.0f} minutes ({mins/60:.2f} hours)")

print("\nCNFI BEHAVIOR:")

print("- CNFI is a sensor, not compute")

print("- Enabled only when AI is active (FULL / THROTTLED)")

print("- Explicitly gated OFF in SEP safe mode")

print("\nNOTES:")

print("- Radiation thresholds are deterministic control points")

print("- SEP model is conservative and non-probabilistic")

print("- Power accounting is explicit and complete")

print("=" * 80)

# ---------------- Visualization ----------------

fig, ax1 = plt.subplots(figsize=(12, 6))

ax1.plot(minutes / 60, radiation, lw=2)

ax1.set_yscale("log")

ax1.set_xlabel("Time (hours)")

ax1.set_ylabel("Radiation Level (relative, log scale)")

ax1.axvspan(

SEP_START_MIN / 60,

(SEP_START_MIN + SEP_DURATION_MIN) / 60,

color="red",

alpha=0.15,

label="SEP Event"

)

ax2 = ax1.twinx()

ax2.plot(minutes / 60, total_power, lw=2, color="orange")

ax2.set_ylabel("Total Power (W)")

fig.legend(loc="upper right")

plt.title("Sentinel v4.6 — Radiation-Adaptive AI & CNFI Power Control")

plt.grid(alpha=0.3)

plt.tight_layout()

plt.savefig("sentinel_v4.6_ai_cnfi_radiation_control_LOCKED.png", dpi=400)

plt.close()

print("✓ Saved: sentinel_v4.6_ai_cnfi_radiation_control_LOCKED.png")

print("STATUS: v4.6 LOCKED")