Sentinel Network – 75% Sat Loss Global Revisit Sim

import numpy as np

# ============================================================

# SENTINEL NETWORK v4.6.1 — GLOBAL REVISIT (HARDENED)

# 75% CATASTROPHIC SATELLITE LOSS SCENARIO

#

# REVISIT DEFINITION:

#   Time since last observation, sampled at each observation

#

# Geometry, LOS, timing, and sampling identical to nominal case

# ============================================================

# ---------------- Mars Parameters ----------------

R_MARS = 3_389_500.0

MU = 4.282837e13

SOL_DURATION = 88642.7

ROT_RATE = 2 * np.pi / SOL_DURATION

# ---------------- Orbit ----------------

ALT = 400_000.0

A_REF = R_MARS + ALT

INC = np.deg2rad(60.0)

# ---------------- Constellation ----------------

NUM_SATS = 168

N_PLANES = 6

SATS_PER_PLANE = NUM_SATS // N_PLANES

DELTA_A = np.linspace(-10.0, 10.0, N_PLANES)

A_PLANES = A_REF + DELTA_A

RAANS = np.deg2rad(np.linspace(0, 360, N_PLANES, endpoint=False))

MEAN_ANOMS = np.linspace(0, 2*np.pi, SATS_PER_PLANE, endpoint=False)

plane_idx = np.repeat(np.arange(N_PLANES), SATS_PER_PLANE)

slot_idx  = np.tile(np.arange(SATS_PER_PLANE), N_PLANES)

# Phase decorrelation (conservative)

PHASE_JITTER = np.random.uniform(0, 2*np.pi, NUM_SATS)

# ---------------- Catastrophic Loss ----------------

LOSS_FRACTION = 0.75

KEEP_FRACTION = 1.0 - LOSS_FRACTION

rng = np.random.default_rng(seed=42)  # deterministic

keep_mask = rng.random(NUM_SATS) < KEEP_FRACTION

print(f"Catastrophic loss applied: {np.sum(keep_mask)}/{NUM_SATS} satellites retained")

# ---------------- Sensor ----------------

FOV_HALF_ANGLE = np.deg2rad(75.0)

COS_FOV = np.cos(FOV_HALF_ANGLE)

# ---------------- Time ----------------

DT = 10.0

DURATION = 2 * SOL_DURATION

STEPS = int(DURATION / DT)

BURN_IN = int(SOL_DURATION / DT)

# ---------------- Surface Grid ----------------

LAT = np.deg2rad(np.linspace(-90, 90, 18))

LON = np.deg2rad(np.linspace(-180, 180, 36))

LAT_G, LON_G = np.meshgrid(LAT, LON, indexing="ij")

surface = np.stack([

R_MARS * np.cos(LAT_G) * np.cos(LON_G),

R_MARS * np.cos(LAT_G) * np.sin(LON_G),

R_MARS * np.sin(LAT_G)

], axis=-1).reshape(-1, 3)

# ---------------- Storage ----------------

last_seen = np.full(len(surface), np.nan)

revisit_samples = []

# ---------------- Helpers ----------------

def rotate_surface(vecs, t):

th = ROT_RATE * t

c, s = np.cos(th), np.sin(th)

R = np.array([[ c,  s, 0],

[-s,  c, 0],

[ 0,  0, 1]])

return vecs @ R.T

def sat_positions(t):

A = A_PLANES[plane_idx]

n = np.sqrt(MU / A**3)

M = n * t + MEAN_ANOMS[slot_idx] + PHASE_JITTER

raan = RAANS[plane_idx]

cM, sM = np.cos(M), np.sin(M)

y_inc = sM * np.cos(INC)

z_inc = sM * np.sin(INC)

x = A * (cM*np.cos(raan) - y_inc*np.sin(raan))

y = A * (cM*np.sin(raan) + y_inc*np.cos(raan))

z = A * z_inc

pos = np.stack((x, y, z), axis=1)

return pos[keep_mask]

def los_clear(sats, surf):

d = surf[None,:,:] - sats[:,None,:]

t = -np.einsum('sd,spd->sp', sats, d) / np.einsum('spd,spd->sp', d, d)

t = np.clip(t, 0.0, 1.0)

closest = sats[:,None,:] + t[:,:,None] * d

return np.linalg.norm(closest, axis=-1) > R_MARS

# ---------------- Main Loop ----------------

for step in range(STEPS):

t = step * DT

sats = sat_positions(t)

surf = rotate_surface(surface, t)

# Horizon / LOS

los_ok = los_clear(sats, surf)

# Nadir FOV

rho = surf[None,:,:] - sats[:,None,:]

rho_hat = rho / np.linalg.norm(rho, axis=-1, keepdims=True)

sat_hat = sats / np.linalg.norm(sats, axis=1)[:,None]

alpha_ok = -np.einsum('spd,sd->sp', rho_hat, sat_hat) > COS_FOV

covered = np.any(los_ok & alpha_ok, axis=0)

if step >= BURN_IN:

seen_before = ~np.isnan(last_seen)

revisit_samples.extend(t - last_seen[covered & seen_before])

last_seen[covered] = t

# ---------------- Results ----------------

revisit = np.array(revisit_samples) / 60.0

print("\n=== SENTINEL NETWORK v4.6.1 — GLOBAL REVISIT (75% LOSS) ===")

print(f"Mean global revisit:        {np.mean(revisit):.2f} minutes")

print(f"95th percentile revisit:   {np.percentile(revisit,95):.2f} minutes")

print(f"99th percentile revisit:   {np.percentile(revisit,99):.2f} minutes")

print("Geometry:                  Sphere-occluded LOS + nadir")

print("Phasing:                   Passive Δa + phase decorrelation")

print("Mars timing:               Exact sidereal rotation")

print("=========================================================")