Sentinel Network – Final Constellation

import numpy as np

import matplotlib.pyplot as plt

# ============================================================

# View 3 — Final Operational Walker Constellation (LOCKED)

# SENTINEL NETWORK v4.6

#

# PURPOSE:

#   Visualize final on-orbit spatial geometry of the Sentinel

#   constellation using a Walker-style distribution.

#

# SCOPE (EXPLICIT):

#   • ORBITAL GEOMETRY ONLY

#   • Payload-agnostic (no CNFI / sensors / AI modeled)

#   • No coverage, power, or operational logic implied

#

# IMPORTANT:

#   This is NOT a coverage or revisit simulation.

#   This view exists solely to illustrate constellation layout.

# ============================================================

# ---------------- Mars Parameters ----------------

R_MARS = 3_389_500          # m (mean Mars radius)

ALTITUDE = 400_000          # m (circular operational altitude)

ORBIT_RADIUS = R_MARS + ALTITUDE

# ---------------- Constellation Definition ----------------

N_PLANES = 6

SATS_PER_PLANE = 28

TOTAL_SATS = N_PLANES * SATS_PER_PLANE

INCLINATION = np.radians(60.0)  # v4.6 locked inclination

# Walker-style plane spacing

plane_raan = np.linspace(0.0, 2.0 * np.pi, N_PLANES, endpoint=False)

true_anomaly = np.linspace(0.0, 2.0 * np.pi, SATS_PER_PLANE, endpoint=False)

# ---------------- Generate Satellite Positions ----------------

sat_positions = []

for raan in plane_raan:

for ta in true_anomaly:

# Perifocal coordinates (circular orbit)

x = ORBIT_RADIUS * np.cos(ta)

y = ORBIT_RADIUS * np.sin(ta)

z = 0.0

# Apply inclination

xi = x

yi = y * np.cos(INCLINATION)

zi = y * np.sin(INCLINATION)

# Apply RAAN rotation

xr = xi * np.cos(raan) - yi * np.sin(raan)

yr = xi * np.sin(raan) + yi * np.cos(raan)

zr = zi

sat_positions.append([xr, yr, zr])

sat_positions = np.array(sat_positions)

# ---------------- Visualization ----------------

fig = plt.figure(figsize=(10, 10), facecolor='black')

ax = fig.add_subplot(111, projection='3d')

ax.set_facecolor('#05070d')

# Mars sphere (reference only)

u = np.linspace(0.0, 2.0 * np.pi, 60)

v = np.linspace(0.0, np.pi, 30)

xm = R_MARS * np.outer(np.cos(u), np.sin(v))

ym = R_MARS * np.outer(np.sin(u), np.sin(v))

zm = R_MARS * np.outer(np.ones_like(u), np.cos(v))

ax.plot_surface(

xm, ym, zm,

color='#b24a2a',

alpha=0.9,

linewidth=0

)

# Satellite markers

ax.scatter(

sat_positions[:, 0],

sat_positions[:, 1],

sat_positions[:, 2],

s=18,

color='#7fffd4',

alpha=0.95

)

# ---------------- Formatting ----------------

ax.set_title(

"View 3 — Final Operational Mars Constellation\n"

"Walker 168:6:1 · 400 km · 60° Inclination",

color='white',

fontsize=15,

weight='bold',

pad=20

)

ax.set_box_aspect([1, 1, 1])

ax.axis('off')

plt.tight_layout()

plt.savefig(

"view3_final_operational_constellation_v4.6.png",

dpi=400,

facecolor='black'

)

plt.close()

print("✓ View 3 LOCKED — Payload-agnostic Walker constellation geometry")