Sentinel Network – Propellant Script

import numpy as np

import matplotlib.pyplot as plt

# ===================================================================

# SENTINEL NETWORK v4.6 — PROPULSION-LIMITED SERVICE LIFE (FINAL LOCKED)

#

# PURPOSE:

#   Quantify xenon sufficiency for long-term orbital maintenance plus

#   one-time formation correction following passive Mars J2 phasing.

#

# IMPORTANT:

#   • PROPULSION-LIMITED ONLY.

#   • This does NOT define mission lifetime.

#   • Power, radiation, avionics, and obsolescence dominate real ops life.

#

# TRACEABILITY:

#   • Fully consistent with Sentinel v4.6 FINAL LOCKED PDR baseline

#   • Δv usage intentionally DERATED for margin, storms, and uncertainty

# ===================================================================

# ----------------------- Constellation Parameters -----------------------

TOTAL_SATS = 168

# ----------------------- Mass Model (v4.6 FINAL) ------------------------

DRY_MASS_BASELINE_KG = 362.0     # Baseline spacecraft (no propulsion)

PROP_DRY_KG          = 30.0      # Thruster + PPU + tank

XENON_MASS_KG        = 50.0      # Optional EP allocation (per satellite)

WET_MASS_INITIAL_KG  = DRY_MASS_BASELINE_KG + PROP_DRY_KG + XENON_MASS_KG

# ----------------------- Propulsion Parameters --------------------------

ISP_SEC = 2200.0                # Ion / gridded thruster heritage

G0      = 9.80665

# ----------------------- Rocket Equation -------------------------------

def propellant_for_delta_v(delta_v, mass_initial):

ve = ISP_SEC * G0

mass_final = mass_initial * np.exp(-delta_v / ve)

return mass_initial - mass_final

def delta_v_capability(mass_initial, prop_mass):

ve = ISP_SEC * G0

return ve * np.log(mass_initial / (mass_initial - prop_mass))

# ----------------------- Total Δv Capability ---------------------------

TOTAL_DELTA_V_AVAILABLE = delta_v_capability(

WET_MASS_INITIAL_KG,

XENON_MASS_KG

)

# ----------------------- Δv Budget Philosophy --------------------------

# NOTE:

#   Total theoretical Δv from 50 kg Xe ≈ TOTAL_DELTA_V_AVAILABLE.

#   Sentinel v4.6 intentionally plans to use <50% of this value.

#

#   RATIONALE FOR DERATING:

#   - Dust storm uncertainty

#   - Atmospheric variability

#   - Thruster aging

#   - Long-term contingency margin

#   - Infrastructure-grade conservatism

FORMATION_CORRECTION_DELTA_V = 5.0     # m/s (post-J2 convergence upper bound)

# Annual operational Δv (Mars orbiter heritage; conservative)

STATION_KEEPING_M_S_PER_YR = 3.0

COLLISION_AVOID_M_S_PER_YR = 1.5

PROGRAMMATIC_MARGIN_M_S    = 1.0

NOMINAL_DELTA_V_PER_YEAR = (

STATION_KEEPING_M_S_PER_YR +

COLLISION_AVOID_M_S_PER_YR +

PROGRAMMATIC_MARGIN_M_S

)

# Deterministic dust storm model (PDR reproducible)

STORM_EXTRA_DELTA_V = 15.0       # m/s per event

STORM_PERIOD_YEARS  = 2          # One storm every 2 years (conservative)

# ----------------------- Context Reporting -----------------------------

print(f"Per-satellite wet mass:        {WET_MASS_INITIAL_KG:.1f} kg")

print(f"Xenon allocation:              {XENON_MASS_KG:.1f} kg")

print(f"Theoretical Δv capability:     {TOTAL_DELTA_V_AVAILABLE:.0f} m/s")

print(f"Nominal annual Δv budget:      {NOMINAL_DELTA_V_PER_YEAR:.1f} m/s")

print(f"Storm penalty:                 {STORM_EXTRA_DELTA_V:.1f} m/s every {STORM_PERIOD_YEARS} yrs\n")

# ----------------------- Formation Correction --------------------------

prop_used_formation = propellant_for_delta_v(

FORMATION_CORRECTION_DELTA_V,

WET_MASS_INITIAL_KG

)

remaining_xe = XENON_MASS_KG - prop_used_formation

current_mass = WET_MASS_INITIAL_KG - prop_used_formation

print(f"Formation correction Δv:       {FORMATION_CORRECTION_DELTA_V:.1f} m/s")

print(f"Xe used (formation):           {prop_used_formation:.2f} kg")

print(f"Xe remaining for operations:   {remaining_xe:.2f} kg\n")

# ----------------------- Operational Lifetime Simulation ----------------

MAX_YEARS = 50

years = []

annual_delta_v = []

cumulative_xe = [prop_used_formation]

for year in range(1, MAX_YEARS + 1):

dv_year = NOMINAL_DELTA_V_PER_YEAR

if year % STORM_PERIOD_YEARS == 0:

dv_year += STORM_EXTRA_DELTA_V

xe_used = propellant_for_delta_v(dv_year, current_mass)

if xe_used > remaining_xe:

break

years.append(year)

annual_delta_v.append(dv_year)

remaining_xe -= xe_used

current_mass -= xe_used

cumulative_xe.append(cumulative_xe[-1] + xe_used)

service_life_years = len(years)

total_xe_used = cumulative_xe[-1]

# ----------------------- Cost Context ----------------------------------

XENON_COST_PER_KG = 10_000.0     # $/kg (space-qualified, conservative)

total_xenon_cost_musd = (TOTAL_SATS * XENON_MASS_KG * XENON_COST_PER_KG) / 1e6

# ----------------------- Results ---------------------------------------

print("=" * 78)

print("SENTINEL NETWORK v4.6 — PROPULSION-LIMITED SERVICE LIFE (FINAL)")

print("=" * 78)

print(f"Propulsion-limited lifetime:    {service_life_years} years")

print(f"Total Xe used per satellite:    {total_xe_used:.2f} kg")

print(f"Unused Xe margin:               {XENON_MASS_KG - total_xe_used:.2f} kg")

print(f"Fraction of Δv utilized:        {100 * total_xe_used / XENON_MASS_KG:.1f}%")

print(f"Constellation xenon cost:       ~${total_xenon_cost_musd:.1f}M")

print("NOTE: Lifetime limited by propulsion ONLY.")

print("      Power, radiation, avionics determine real mission life.")

print("=" * 78)

# ----------------------- Visualization ---------------------------------

plt.figure(figsize=(10, 6))

plt.plot([0] + years, cumulative_xe, lw=2, label="Cumulative Xenon Used (kg/sat)")

plt.bar(years, annual_delta_v, alpha=0.3, label="Annual Δv (m/s)")

plt.xlabel("Mission Year")

plt.ylabel("Δv / Xenon Usage")

plt.title("Sentinel Network v4.6 — Propulsion-Limited Lifetime (FINAL LOCKED)")

plt.grid(alpha=0.3)

plt.legend()

plt.tight_layout()

plt.savefig("sentinel_propulsion_lifetime_v4.6_FINAL.png", dpi=400)

plt.close()

print("✓ Saved: sentinel_propulsion_lifetime_v4.6_FINAL.png")

s