Sentinel Network – Worst Case Script

import numpy as np

import matplotlib.pyplot as plt

# =============================================================================

# SENTINEL NETWORK v4.6 — WORST-CASE INTEGRATED STRESS TEST (REVISED)

# PDR-GRADE | LOCKED | TRACEABLE

#

# PURPOSE:

#   Bound Sentinel survivability under intentionally stacked,

#   conservative, non-probabilistic failure conditions.

#

# IMPORTANT:

#   This scenario is NOT expected operationally.

#   It demonstrates survivability envelope only.

# =============================================================================

# ---------------- Time Axis ----------------

minutes = np.arange(0, 1440, 1.0)  # 1 sol (minutes)

# ---------------- Constellation Attrition ----------------

INITIAL_SATS   = 168

SURVIVING_SATS = 96                 # ~43% loss (bounding case)

sat_fraction   = SURVIVING_SATS / INITIAL_SATS

# ---------------- Power Baseline (v4.6 locked) ----------------

SOLAR_CLEAN_W = 457.0               # Clean EOL average solar generation

# Nominal spacecraft loads

BUS_AVIONICS_W = 95.0               # Nominal avionics / ADCS / comms

SENSORS_W      = 85.0               # MARCI + EMIRS + Liulin

BASE_LOAD_W    = BUS_AVIONICS_W + SENSORS_W

# Survivability-only bus (safe-mode)

BUS_MIN_W = 149.0                   # Avionics + ADCS + heaters + comms only

# AI + CNFI loads

AI_FULL_W      = 700.0

AI_THROTTLED_W = 350.0

AI_SAFE_W      = 0.0

CNFI_W         = 8.0

# ---------------- Dust Storm ----------------

# Global dust storm heritage (MRO / InSight)

DUST_REMAINING_FRAC = 0.45

solar_power = np.full(len(minutes), SOLAR_CLEAN_W * DUST_REMAINING_FRAC)

# ---------------- Eclipse ----------------

ECLIPSE_START = 700

ECLIPSE_DUR   = 45

solar_power[ECLIPSE_START:ECLIPSE_START + ECLIPSE_DUR] = 0.0

# ---------------- Radiation Environment ----------------

radiation = np.ones(len(minutes))   # GCR baseline = 1×

SEP_START = 500

SEP_DUR   = 180

SEP_MULT  = 500.0                   # 1972-class SEP proxy

radiation[SEP_START:SEP_START + SEP_DUR] *= SEP_MULT

# ---------------- Adaptive Power Logic ----------------

ai_power     = np.zeros(len(minutes))

cnfi_power   = np.zeros(len(minutes))

sensor_power = np.full(len(minutes), SENSORS_W)

total_power  = np.zeros(len(minutes))

for i in range(len(minutes)):

sol = solar_power[i]

rad = radiation[i]

# ---- SAFE MODE ----

# Triggered by extreme SEP or insufficient solar generation

if rad >= 100.0 or sol <= 250.0:

ai_power[i]     = 0.0

cnfi_power[i]   = 0.0

sensor_power[i] = 0.0

total_power[i]  = BUS_MIN_W

# ---- DEGRADED MODE ----

elif rad >= 10.0:

ai_power[i]     = AI_THROTTLED_W * sat_fraction

cnfi_power[i]   = CNFI_W * sat_fraction

sensor_power[i] = SENSORS_W * sat_fraction

total_power[i]  = BUS_AVIONICS_W + sensor_power[i] + ai_power[i] + cnfi_power[i]

# ---- NOMINAL MODE ----

else:

ai_power[i]     = AI_FULL_W * sat_fraction

cnfi_power[i]   = CNFI_W * sat_fraction

sensor_power[i] = SENSORS_W * sat_fraction

total_power[i]  = BUS_AVIONICS_W + sensor_power[i] + ai_power[i] + cnfi_power[i]

# ---------------- Battery Model ----------------

# Nominal usable ~12 kWh; derated to 10 kWh for compound stress

BAT_WH = 10_000.0

energy = BAT_WH

soc = []

for net in (solar_power - total_power) * (1 / 60):  # Wh per minute

energy = np.clip(energy + net, 0.0, BAT_WH)

soc.append(100.0 * energy / BAT_WH)

soc = np.array(soc)

# Survivability assertion

assert soc.min() > 0.0, "Battery depletion occurred — survivability failed"

# ---------------- Report ----------------

print("=" * 78)

print("SENTINEL NETWORK v4.6 — WORST-CASE INTEGRATED STRESS TEST")

print("=" * 78)

print(f"Satellites remaining: {SURVIVING_SATS} ({sat_fraction:.0%})")

print(f"Minimum battery SoC:  {soc.min():.1f}%")

print(f"Maximum DoD:          {100 - soc.min():.1f}%")

print("Result: System survives dust + eclipse + SEP + attrition")

print("Safe-mode: Sensors, AI, and CNFI gated dark when")

print("           solar <250 W or SEP radiation ≥100× GCR")

print("NOTE: Scenario intentionally conservative and non-probabilistic")

print("=" * 78)

# ---------------- Visualization ----------------

fig, ax = plt.subplots(3, 1, figsize=(12, 10), sharex=True)

ax[0].plot(minutes, solar_power, lw=2)

ax[0].set_ylabel("Solar Power (W)")

ax[0].grid(alpha=0.3)

ax[1].plot(minutes, total_power, lw=2)

ax[1].set_ylabel("Total Load (W)")

ax[1].grid(alpha=0.3)

ax[2].plot(minutes, soc, lw=2)

ax[2].set_ylabel("Battery SoC (%)")

ax[2].set_xlabel("Time (minutes)")

ax[2].grid(alpha=0.3)

plt.suptitle("Sentinel v4.6 — Worst-Case Integrated Stress Test (LOCKED)")

plt.tight_layout()

plt.savefig("sentinel_worst_case_v4.6.png", dpi=400)

plt.close()

print("✓ Saved: sentinel_worst_case_v4.6.png")

print("STATUS: v4.6 LOCKED")